Certificate

We hereby confirm the toner cartridge (NCM CODE: 8443 9929) manufactured by MACAO MILLION HONOUR INTERNATION CO. LTD is printer consumable products. The toner cartridges are safety commodities. All the toner in the cartridge were imported from Japan. Please refer to MSDA for the toner.

MATERIAL SAFETY DATA SHEET

Section I Company Name:; Macao Million Honour International Co.Ltd. Address:; ZHUHAI,CHINA. Product Type:; TONER.

| HAZARDOUS INGREDIENTS / IDENTITY INFORMATION |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Section II |  |  |  |  |  |  |  |
| Hazardous Components ( Chemical Identity : Common Names ) OSHA PEL |  |  | ACGIH TLV |  | ( Optional )% |  |  |
| Styrene Acrylate Copolymer | not listed |  | not listed | >85% |  |  |  |
| Carbon Black | 3.5mg /m3 |  | 3.5mg /m3 | <9% |  |  |  |
| Silica | 15mg /m3 |  | 10mg /m3 | <3% |  |  |  |
| Polyolefin Wax | not listed |  | not listed | <3% |  |  |  |
| Dye | not listed |  | not listed | <3% |  |  |  |
| PHYSICAL / CHEMICAL CHARACTERISTICS |  |  |  |  |  |  |  |
| Section III |  |  |  |  |  |  |  |
| Boiling Point (F): |  | Not Applicable |  |  |  |  |  |
| Specific Gravity (Water = 1): |  | Not Applicable |  |  |  |  |  |
| Vapor Pressure (mm HG): |  | Not Applicable |  |  |  |  |  |
| Melting Point: |  | > 100 Deg C |  |  |  |  |  |
| Vapor Density (Air = 1): |  | Not Applicable |  |  |  |  |  |
| Evaporation Rate (Butyl Acetate =1): |  | Not Applicable |  |  |  |  |  |
| Solubility in Water: |  | Insoluble |  |  |  |  |  |
| Appearance & Odor: |  | Fine black powder with slight odor. |  |  |  |  |  |
| FIRE & EXPLOSION HAZARD DATA |  |  |  |  |  |  |  |
| Section IV |  |  |  |  |  |  |  |
| Flash Point (Method Used): |  | Not Applicable |  |  |  |  |  |
| Flammable Limits: |  | Not Applicable |  |  |  |  |  |
| LEL: |  | Not Applicable |  |  |  |  |  |
| UEL: |  | Not Applicable |  |  |  |  |  |
| Extinguishing Media: |  | Water Fog, Foam , Dry Chemical, Co2 |  |  |  |  |  |
| Special Fire Fighting Procedures: |  | Avoid inhalation of smoke , Wear MSHA / NIOSH S.C.U.B.A |  |  |  |  |  |
| Unusual Fire and Explosion Hazards: |  | Toner is a combustible powder. When dispersed in air, it forms explosive mixtures. Avoid all Ignite Sources |  |  |  |  |  |
| . |  |  |  |  |  |  |  |
| Reactive Data |  |  |  |  |  |  |  |
| Section V |  |  |  |  |  |  |  |
| Stability: |  |  | Unstable: |  | Stable: |  | X |
| Conditions to Avoid: |  | No Data |  |  |  |  |  |
| Incompatibility (Materials to Avoid): |  | None |  |  |  |  |  |
| Hazardous Decomposition or Byproducts: |  | Only after combustion |  |  |  |  |  |
| Hazardous Polymerization: |  |  | Will Occur: |  | Will Not Occur: |  | X |

| HEALTH HAZARD DATA |  |  |  |  |
| --- | --- | --- | --- | --- |
| Section VI |  |  |  |  |
| Routes of Entry: |  |  |  |  |
| Eyes: |  |  | Possible |  |
| Inhalation: |  |  | Possible |  |
| Skin: |  |  | None Known |  |
| Ingestion: |  |  | Possible |  |
| Health Hazards (Acute or Chronic): |  |  |  | None Known |
| Signs or Symptoms of Exposure: |  |  |  | Respiratory irritation in prolonged exposure to excessive dust. |
| Medical Conditions Aggravated by Exposure: |  |  |  | None Known |
| Carcinogenicity: | Carbon Black was reclassified as a group 2B by IARC in 1996 based on the result of only the inhalation study in ruts. However there was not observed the incidence of tumors on that results on dermal or oral studies. Also 2-years inhalation study using a typical toner containing carbon black showed no association between toner exposure and animal tumors. |  |  |  |
| EMERGENCY AND FIRST AID PROCEDURES: |  |  |  |  |
| EYES: |  |  | Flush with water. |  |
| INHALATION: |  |  | Remove from exposure. |  |
| SKIN: |  |  | Wash with soap and water. |  |
| INGESTION: |  |  | Dilute stomach contents with several glasses of water. |  |
| PRECAUTIONS FOR SAFE HANDLING AND USE |  |  |  |  |
| Section VII |  |  |  |  |
| Steps to be Taken in Case Material is Released: |  |  |  | Loose toner can be removed by using vacuum cleaner or broom. Residue can be removed with soap and water. |
| Waste Disposal Method: |  |  |  | Do not incinerate. Dispose of in accordance with federal, state, and local regulations. |
| Precautions to be Taken in Handling or Storage: |  |  |  | Eye protection and respirators may be required for bulk handling. Avoid storing next to strong oxidizers. |
| Other Precautions: |  |  |  | Avoid excessive exposure to heat |
| CONTROL MEASURES |  |  |  |  |
| Section VIII |  |  |  |  |
| Respiratory Protection: |  |  |  | Follow Federal and State guide lines for respirator selection and use. |
| Ventilation: |  |  |  |  |
| Local: |  | Local exhaust ventilation is preferred for controlling dispersion at source |  |  |
| Mechanical: |  | No Data |  |  |
| Special: |  | No Data |  |  |
| Other: |  | Provide general or local exhaust ventilation to maintain below OSHA PELs |  |  |
| Protective Gloves: |  |  |  | Wear chemically protective gloves, boots, aprons and gauntlets |
| Eye Protection: |  |  |  | Wear protective eyeglasses or chemical safety goggles |
| Other Protective Clothing or Equipment: |  |  |  | None required. |
| Work / Hygienic Practices: |  |  |  | Wash hands with soap and water prior to eating. |