| Inkjet ink, washable:Cont Jet (175/1) |  | 23.036.GBR |
| --- | --- | --- |
| IDENTIFICATION OF THE SUBSTANCE / PREPARATION AND OF THE COMPANY / UNDERTAKING |  |  |
| DISTRIBUTOR: Pelikan Hardcopy European Logistics & Services GmbH Neue Strasse 19 D-52382 Niederzier 2 Germany Telephone: +49 (0) 2428 830 ADMIN/TECH CENTRE: Pelikan Hardcopy (International) AG Gewerbestrasse 9 CH-8132 Egg/ZH Switzerland Telephone: +41 (0) 1 986 1111 Emergency Telephone: +41 (0) 1 986 1111 |  |  |

SAFETY DATA SHEET

| PRODUCT NAME: Inkjet ink, washable:Cont Jet (175/1) | SDS ref: 23.036.GBR |
| --- | --- |

| 2 | COMPOSITION / INFORMATION ON INGREDIENTS |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Ingredients | CAS No. | EC No. | Weight % | Symbols | R Phrases |
| ### | butanone | 78-93-3 | 201-159-0 | 45-50 | F,Xi | 11,36,66,67 |
|  | ethanol | 64-17-5 | 200-578-6 | 25-30 | F | 11 |

| 3 | HAZARDS IDENTIFICATION |  |
| --- | --- | --- |
| ### | - Highly Flammable (R11) - Irritating to eyes (R36) - Repeated exposure may cause skin dryness or cracking (R66) - Vapours may cause drowsiness and dizziness (R67) |  |
| 4 | FIRST-AID MEASURES |  |
|  | CONTACT WITH SKIN |  |
|  | - Wash affected area with plenty of soap and water |  |
|  | CONTACT WITH EYES |  |
| ### | - In case of contact with eyes, rinse immediately with plenty of water and seek medical advice (S26) |  |
|  | INGESTION |  |
|  | - Give water or milk to drink - Seek immediate medical attention |  |
|  | INHALATION |  |
|  | - Remove patient to fresh air - Keep warm and at rest |  |
|  | OTHER INFORMATION |  |
|  | - See Section 11 - Toxicological Information - When in doubt or symptoms persist, seek medical attention |  |
| 5 | FIRE-FIGHTING MEASURES |  |
|  | - In case of fire use water, carbon dioxide or dry agent (S43) - Do not use water jets - Toxic and irritant fumes may be formed |  |
| Revision B dated 21.05.01 |  | Page 1 of 4 |

| Inkjet ink, washable:Cont Jet (175/1) |  |  | 23.036.GBR |
| --- | --- | --- | --- |
| 6 | ACCIDENTAL RELEASE MEASURES |  |  |
|  | - Shut off all ignition sources - Do not allow to enter public sewers and watercourses - Ventilate area - Absorb spillage in suitable inert material - See Section 13 |  |  |
| 7 | HANDLING AND STORAGE |  |  |
| 7.1 | HANDLING |  |  |
|  | - Keep away from heat and sources of ignition - Avoid contact with eyes - The usual precautions for handling chemicals should be observed |  |  |
| 7.2 | STORAGE |  |  |
| ### | - Keep container in a well ventilated place (S9) - Keep away from sources of ignition - No Smoking (S16) - Keep in highly flammable materials store |  |  |
| 8 | EXPOSURE CONTROLS / PERSONAL PROTECTION |  |  |
| 8.1 | GENERAL |  |  |
|  | - Wear plastic gloves - Use good personal hygiene practices |  |  |
| 8.2 | EXPOSURE LIMITS | Constituent Exposure Limit: |  |
| ### | - butanone | OES / LTEL 200 ppm 600 mg/m3 OES / STEL 300 ppm 899 mg/m3 (Germany) MAK 200 ppm 600 mg/m3 Can be absorbed through the skin |  |
| ### | - ethanol | OES / LTEL 1000 ppm 1920 mg/m3 OES / STEL --- ppm --- mg/m3 (Germany) MAK 500 ppm 960 mg/m3 |  |
| 9 | PHYSICAL AND CHEMICAL PROPERTIES |  |  |
|  | - Liquid, black, mobile, dispersible in water - Unpleasant odour - Lower explosive limit 1.8 % (in air) - Upper explosive limit 11.5 % (in air) - Product increases fire risk - Combustible - Boiling point ca 80 deg C - Flash point ca -4 deg C (CC) - pH 6.8 at 50 % concentration - Density 0.879 g/cm3 at 25 deg C - Vapour pressure ca 69.7 mbar - Water solubility > 600 g/l |  |  |
| 10 | STABILITY AND REACTIVITY |  |  |
|  | - Keep away from heat - Keep away from strong oxidizing substances - Decomposition products may include nitrogen and carbon oxides |  |  |
| Revision B dated 21.05.01 |  |  | Page 2 of 4 |

| Inkjet ink, washable:Cont Jet (175/1) |  |  |  |  |  | 23.036.GBR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 11 | TOXICOLOGICAL INFORMATION |  |  |  |  |  |  |  |
| ### | - Inhalation of solvent vapours may give rise to nausea, headaches and dizziness - Prolonged skin contact will result in defatting of the skin, leading to irritation, and in some cases, dermatitis - Vapours or aerosols may cause irritation of eyes, nose and respiratory tract - May cause gastrointestinal disturbances |  |  |  |  |  |  |  |
| 12 | ECOLOGICAL INFORMATION |  |  |  |  |  |  |  |
|  | - Water Hazard Class 1 (Company) |  |  |  |  |  |  |  |
| 13 | DISPOSAL CONSIDERATIONS |  |  |  |  |  |  |  |
| ### | - Disposal should be in accordance with local, state or national legislation - This material and its container must be disposed of as hazardous waste (S60) - Disposal can be a hazardous operation, seek specialist advice |  |  |  |  |  |  |  |
| 14 | TRANSPORT INFORMATION |  |  |  |  |  |  |  |
| 14.1 | REGULATIONS |  |  |  |  |  |  |  |
|  | - Proper Shipping Name: Printing Ink |  |  |  |  |  |  |  |
|  | UN |  |  |  |  |  |  |  |
|  |  | UN No | : | 1210 | Hazard Class |  | : | 3 |
|  |  | Packing Group | : | II |  |  |  |  |
|  | Road / Rail (ADR / RID) |  |  |  |  |  |  |  |
| ### |  | ADR UN No | : | 1210 | ADR Hazard Class |  | : | 3 |
|  |  | ADR Item No | : | 5(b) | ADR/RID No |  | : | HZ33 |
|  |  | Tremcard | : | ND |  |  |  |  |
|  | Sea (IMDG) |  |  |  |  |  |  |  |
| ### |  | IMDG UN No | : | 1210 | IMDG Hazard Class |  | : | 3.2 |
|  |  | IMDG Page No | : | 3272-1 | IMDG Pack. Group |  | : | II |
|  |  | IMDG EmS | : | 3-05 | IMDG MFAG |  | : | 311 |
|  | Air (ICAO / IATA) |  |  |  |  |  |  |  |
|  |  | ICAO UN No | : | 1210 | ICAO Hazard Class |  | : | 3 |
|  |  | ICAO Packing Group | : | II |  |  |  |  |
| 14.2 | GENERAL |  |  |  |  |  |  |  |
|  | - Transport Regulations apply as indicated in previous section |  |  |  |  |  |  |  |
| Revision B dated 21.05.01 |  |  |  |  |  | Page 3 of 4 |  |  |

| Inkjet ink, washable:Cont Jet (175/1) |  |  |  |  | 23.036.GBR |
| --- | --- | --- | --- | --- | --- |
| 15 | REGULATORY INFORMATION |  |  |  |  |
|  | - This product is covered by Dangerous Preparations Directive (88/379/EEC) as amended - Refer to current CHIP Regulations - Refer to current Gefahrliche Stoffe Verordnung - Refer to current VbF - This Safety Data Sheet is provided in compliance with the Safety Data Sheets Directive (91/155/EC) |  |  |  |  |
|  | SYMBOL(S) |  |  |  |  |
| 15.1 | RISK PHRASES |  |  |  |  |
| ### | - Highly Flammable (R11) - Irritating to eyes (R36) - Repeated exposure may cause skin dryness or cracking (R66) - Vapours may cause drowsiness and dizziness (R67) |  |  |  |  |
| 15.2 | SAFETY PHRASES |  |  |  |  |
| ### | - Keep container in a well ventilated place (S9) - Keep away from sources of ignition - No Smoking (S16) - In case of contact with eyes, rinse immediately with plenty of water and seek medical advice (S26) - This material and its container must be disposed of as hazardous waste (S60) |  |  |  |  |
| 16 | # OTHER INFORMATION |  |  |  |  |
|  | - The precise composition of this product is proprietary information. More details will be supplied in confidence to qualified medical personnel if specific treatment is necessary. - This product conforms to all known Euro chemicals safety legislation. - Water Hazard Class: 1 = slight, 2 = medium, 3 = severe hazard. - NA = not applicable; NC = not classified; ND = no data; TWA = time-weighted average. - LTEL = 8hr TWA; STEL = 15min TWA; ppm = parts per million. - ### = changes to section since last revision. - The above information relates only to the safety of the product designated, based on our current state of knowledge. No responsibility is accepted for any errors or omissions. |  |  |  |  |
| Revision B dated 21.05.01 |  |  |  |  | Page 4of 4 |