| Inkjet inks:Kodak Diconix 330C (118/1;119/1;120/0) |  | 22.026.GBR |
| --- | --- | --- |
| IDENTIFICATION OF THE SUBSTANCE / PREPARATION AND OF THE COMPANY / UNDERTAKING |  |  |
| DISTRIBUTOR: Pelikan Hardcopy European Logistics & Services GmbH Neue Strasse 19 D-52382 Niederzier 2 Germany Telephone: +49 (0) 2428 830 ADMIN/TECH CENTRE: Pelikan Hardcopy (International) AG Gewerbestrasse 9 CH-8132 Egg/ZH Switzerland Telephone: +41 (0) 1 986 1111 Emergency Telephone: +41 (0) 1 986 1111 |  |  |

SAFETY DATA SHEET

| PRODUCT NAME : Inkjet inks:Kodak Diconix 330C (118/1;119/1;120/0) | SDS ref: 22.026.GBR |
| --- | --- |

| 2 | COMPOSITION / INFORMATION ON INGREDIENTS |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Ingredients | CAS No. | EC No. | Weight % | Symbols | R Phrases |
| ### | diethylene glycol | 111-46-6 | 203-872-2 | 40-45 | Xn | 22 |
| ### | N-methyl-2-pyrrolidone | 872-50-4 | 212-828-1 | 5-10 | Xi | 36/38 |
| ### | Propan-1-ol | 71-23-8 | 200-746-9 | 5-10 | F,Xi | 11,41,67 |

| 3 | HAZARDS IDENTIFICATION |  |
| --- | --- | --- |
|  | - Harmful if swallowed (R22) - Irritating to eyes and skin (R36/38) - Can be absorbed through skin - See Section 4 |  |
| 4 | FIRST-AID MEASURES |  |
|  | CONTACT WITH SKIN |  |
|  | - Wash affected area with plenty of soap and water - Seek medical attention if irritation persists |  |
|  | CONTACT WITH EYES |  |
| ### | - In case of contact with eyes, rinse immediately with plenty of water and seek medical advice (S26) |  |
|  | INGESTION |  |
|  | - Give water or milk to drink - If swallowed, seek medical advice immediately and show this container or label (S46) |  |
|  | INHALATION |  |
|  | - Remove patient to fresh air - Keep warm and at rest |  |
|  | OTHER INFORMATION |  |
|  | - See Section 11 - Toxicological Information |  |
| 5 | FIRE-FIGHTING MEASURES |  |
|  | - In case of fire use water, carbon dioxide or dry agent (S43) - Do not use water jets - May give off noxious and toxic fumes in a fire |  |
| Revision D dated 16.05.01 |  | Page 1 of 4 |

| Inkjet inks:Kodak Diconix 330C (118/1;119/1;120/0) |  |  | 22.026.GBR |
| --- | --- | --- | --- |
| 6 | ACCIDENTAL RELEASE MEASURES |  |  |
|  | - Shut off all ignition sources - Do not allow to enter public sewers and watercourses - Ventilate area - Absorb spillage in suitable inert material - See Section 13 |  |  |
| 7 | HANDLING AND STORAGE |  |  |
| 7.1 | HANDLING |  |  |
|  | - Keep away from heat and sources of ignition - Avoid contact with combustible material - Avoid contact with skin & eyes - The usual precautions for handling chemicals should be observed |  |  |
| 7.2 | STORAGE |  |  |
|  | - Keep in a cool, dry, well ventilated place - Keep out of reach of children (S2) - Keep away from food, drink and animal feeding stuffs (S13) |  |  |
| 8 | EXPOSURE CONTROLS / PERSONAL PROTECTION |  |  |
| 8.1 | GENERAL |  |  |
|  | - Wear plastic gloves - Use good personal hygiene practices |  |  |
| 8.2 | EXPOSURE LIMITS | Constituent Exposure Limit: |  |
| ### | - Diethylene glycol | OES / LTEL 23 ppm 101 mg/m3 OES / STEL --- ppm --- mg/m3 (Germany) MAK 10 ppm 44 mg/m3 |  |
| ### | - N-methyl-2-pyrrolidone | OES / LTEL 25 ppm 103 mg/m3 OES / STEL 75 ppm 309 mg/m3 (Germany) MAK 19 ppm 80 mg/m3 Can be absorbed through the skin |  |
|  | - Propan-1-ol | OES / LTEL 200 ppm 500 mg/m3 OES / STEL 250 ppm 625 mg/m3 (France) VLE 200 ppm 500 mg/m3 Can be absorbed through the skin |  |
| 9 | PHYSICAL AND CHEMICAL PROPERTIES |  |  |
|  | - Liquid, coloured, miscible with water - Perceptible odour - May form explosive mixtures with air - Combustible - Boiling point ND deg C - Flash point ca 67 deg C (CC) - pH 6.7-7.6 - Density 1.06-1.07 g/cm3 at 25 deg C - Vapour pressure ca 19 mbar |  |  |
| Revision D dated 16.05.01 |  |  | Page 2 of 4 |

| Inkjet inks:Kodak Diconix 330C (118/1;119/1;120/0) |  |  |  |  | 22.026.GBR |
| --- | --- | --- | --- | --- | --- |
| 10 | STABILITY AND REACTIVITY |  |  |  |  |
|  | - Keep away from heat - Keep away from strong oxidizing substances - Decomposition products may include nitrogen and carbon oxides |  |  |  |  |
| 11 | TOXICOLOGICAL INFORMATION |  |  |  |  |
| ### | - Irritating to eyes (R36) - Can be absorbed through skin - Harmful if swallowed (R22) - May cause gastrointestinal disturbances |  |  |  |  |
| 12 | ECOLOGICAL INFORMATION |  |  |  |  |
|  | - Water Hazard Class 1 (Company) |  |  |  |  |
| 13 | DISPOSAL CONSIDERATIONS |  |  |  |  |
| ### | - Disposal should be in accordance with local, state or national legislation - Dispose of this material and its container at hazardous or special waste collection point (S56) - Disposal can be a hazardous operation, seek specialist advice |  |  |  |  |
| 14 | TRANSPORT INFORMATION |  |  |  |  |
| 14.1 | REGULATIONS |  |  |  |  |
|  | - Proper Shipping Name: NA |  |  |  |  |
| 14.2 | GENERAL |  |  |  |  |
|  | - Not classified as hazardous for transport |  |  |  |  |
| 15 | REGULATORY INFORMATION |  |  |  |  |
|  | - This product is covered by Dangerous Preparations Directive (88/379/EEC) as amended - Refer to current CHIP Regulations - Refer to current Gefahrliche Stoffe Verordnung - This Safety Data Sheet is provided in compliance with the Safety Data Sheets Directive (91/155/EC) |  |  |  |  |
|  | SYMBOL(S) |  |  |  |  |
| 15.1 | RISK PHRASES |  |  |  |  |
| ### | - Harmful if swallowed (R22) - Irritating to eyes (R36) |  |  |  |  |
| 15.2 | SAFETY PHRASES |  |  |  |  |
| ### | - Keep out of reach of children (S2) - In case of contact with eyes, rinse immediately with plenty of water and seek medical advice (S26) - If swallowed, seek medical advice immediately and show this container or label (S46) - Dispose of this material and its container at hazardous or special waste collection point (S56) |  |  |  |  |
| Revision D dated 16.05.01 |  |  |  |  | Page 3 of 4 |

| Inkjet inks:Kodak Diconix 330C (118/1;119/1;120/0) |  | 22.026.GBR |
| --- | --- | --- |
| 16 | OTHER INFORMATION |  |
|  | - The precise composition of this product is proprietary information. More details will be supplied in confidence to qualified medical personnel if specific treatment is necessary. - This product conforms to all known Euro chemicals safety legislation. - Water Hazard Class: 0 = none, 1 = slight, 2 = medium, 3 = severe hazard. - NA = not applicable; NC = not classified; ND = no data; TWA = time-weighted average. - LTEL = 8hr TWA; STEL = 15min TWA; ppm = parts per million. - ### = changes to section since last revision. - The above information relates only to the safety of the product designated, based on our current state of knowledge. No responsibility is accepted for any errors or omissions. |  |
| Revision D dated 16.05.01 |  | Page 4 of 4 |