MATERIAL SAFETY DATA SHEET

DPI Solutions, Inc.

The Date of Issue: 3/1/2007	 Version Number: 1.1

SECTION 1 - CHEMICAL PRODUCT AND COMPANY IDENTIFICATION

| Product Code | CMH2600 |
| --- | --- |
| Product Name | Polyester color toner |
| Manufacturer Name | DPI Solutions, Inc. |
| Company Address | 6 Shinsung-dong, Yusung-gu, Deajon, 305-345, KOREA |
| Section | R&D center |
| Manager | Dr. Eui-jun Choi (ejchoi@dpisolutions.net) |
| Company Phone Number | +82-42-865-6912 |
| Company Fax Number | +82-42-865-6915 |

SECTION 2 - COMPOSITION, INFORMATION ON INGREDIENTS

| INGREDIENTS | CAS | %wt | ACGIH TLV | OSHA PEL |
| --- | --- | --- | --- | --- |
| Copolyester of Terephthalic Acid, Isophthalic Acid, n-Propylene Glycol | N/A | < 95% | No | No |
| Colorants – Red Colorants – Blue Colorants – Yellow Colorants – Black | 3648-36-0 26850-47-5 83949-75-1 3648-36-0 26850-47-5 83949-75-1 1333-86-4 | < 3% < 3% < 3% < 5% | No No No No | No No No No |
| CCA | 13170-05-3 19715-19-6 | < 2% < 1% | No No | No No |
| Wax | 8002-74-2 | < 3% | No | No |
| Silica | 68909-20-6 | < 3% | No | No |

SECTION 3 - HAZARD IDENTIFICATION

Hazard to Humans - None during normal use.

Primary Route of Entry - Dermal or inhalation

SECTION 4 - FIRST-AID MEASURES

| INHALATION | Remove from exposure. |
| --- | --- |
| SKIN | Wash skin with soap and water. |
| EYES | Flush eyes with large quantities of water for at least 15 minutes. If irritation is present after washing, contact a physician. |
| INGESTION | Dilute by drinking water. |

SECTION 5 - FIRE-FIGHTING MEASURES

| Hazardous Combustion Products | : CO2, CO, AlOx |
| --- | --- |
| Extinguishing Media | : Water, dry chemicals, foam, carbon dioxide |
| Unusual Fire or Explosion Hazards | : Toner material, like most organic materials in powder form, is capable of creating a dust explosion. |
| Special Fire-Fighting Procedure | : None |

SECTION 6 - ACCIDENTAL RELEASE MEASURES

Spill Procedures:	Collect material without dusting; dispose of as hazardous waste.

Waste Disposal Methods:	Dispose of as flammable, hazardous waste solid, in accordance with 
				Federal, State and Local regulations.

SECTION 7 - HANDLING AND STORAGE

Handling Precautions:

- Handle in accordance with good industrial practice, handle with care and avoid any unnecessary contact.

- Keep away from heat, sparks, or open flame.

- Avoid uncontrolled procedures which may result in generating excessive dust or vapor that can be inhaled or contact the eyes.

- Use adequate local ventilation if dust or vapor generation cannot be avoided.

- Avoid skin contact with this material or solutions of this material.

- Proper housekeeping practices should be maintained.

- Shipping and Storage:

- Keep container tightly closed when not in use.

- Store in a cool, dry place away from heat, spark or open flame. Store away from other combustibles and strong oxidizers.

- Personal Hygiene:

- Practice good personal hygiene.

- Do not eat, smoke or apply cosmetics in areas where the material is being handled.

- Wash thoroughly with soap and water after handling.

- Launder clothing prior to use.

SECTION 8 - EXPOSURE CONTROL/PERSONAL PROTECTION

Engineering Measures - None

Personal Protective Equipment - None

SECTION 9 - PHYSICAL AND CHEMICAL PROPERTIES

| Appearance - fluffy particles Odor and Odor Threshold - None pH - Not applicable Boiling Point - Not applicable Vapor Pressure - Not applicable Vapor Density - Not applicable Melting Point - None but softens at about 60oC Flammable Limits - Not available | Explosive Properties - Not applicable Partition Coefficient - Not applicable Oxidizing Properties - Not applicable Solubility in Water - Insoluble Percent Volatile - None Evaporation Rate - Not applicable Freezing Point - Not applicable Flash Point - Not available |
| --- | --- |

SECTION 10 - STABILITY AND REACTIVITY

Stability: Stable; Conditions to Avoid: Stable in air. Incompatibility (Materials to Avoid): Avoid mixing with strong oxidizing agents. Hazardous Combustion Products: Decomposition products may include toxic phenols. Hazardous Polymerization: Will not occur;

SECTION 11 - TOXICOLOGICAL INFORMATION

ACUTE

Eye Contact:	No substance specific information available.

Polymer particles may act as foreign body.

Skin Contact:	Stains skin and fabrics.

Ingestion:	No applicable information found.

Inhalation:	No applicable information available. Polymer particle may be considered an inert 
			nuisance dust.

CHRONIC

Mutagenicity: ND
	Carcinogenicity: ND
	Other: ND

SECTION 12 - ECOLOGICAL INFORMATION

None

SECTION 13 - DISPOSAL INFORMATION

Waste Disposal - Dispose of used product in accordance with applicable regulations. Do not incinerate.

SECTION 14 - TRANSPORTATION INFORMATION

Proper Shipping Name - None

Hazard Class - None

Identification Number - None

RQ - Not applicable

Packing Group - None

Page Number - None

SECTION 15 - REGULATORY INFORMATION

Follow all regulation in your country.

SECTION 16 – OTHER INFORMATION

None

THE INFORMATION AND RECOMMENDATIONS PROVIDED HEREIN ARE BELIEVED TO BEST OF OUR KNOWLEDGE TO BE ACCURATE. HOWEVER, NEITHER DPI SOLUTIONS, INC. NOR ANY OF ITS AFFILIATES MAKES ANY EXPRESSED OR IMPLIED GUARANTEE OR WARRANTY OF ANY KIND WITH RESPECT TO THE INFORMATION CONTAINED. USER ASSUMES ALL RISKS INCIDENT TO ITS USE.