REVISION DATE:

SAFETY DATA SHEET

SPECIMEN SDS: Toner / Ink for C9352A………………………………………………………………………………

1. IDENTIFICATION OF THE SUBSTANCE/PREPARATION AND OF THE COMPANY/UNDERTAKING

| # PRODUCT NAME | SPECIMEN SDS: Toner / Ink for |
| --- | --- |

| SUPPLIER | Environmental Business Products LTD | # MANUFACTURER |  |
| --- | --- | --- | --- |
| Street | 22 Abbey Rd, Park Royal, London | Street |  |
| Postal Code / City | NW10 7RE | Postal Code / City |  |
| Country | England | Country |  |
| Phone | 44 (0) 20 8838 6888 | Phone |  |
| ## SDS EMAIL CONTACT |  |  |  |
| EMERGENCY TELEPHONE |  |  |  |

# 2. HAZARDS IDENTIFICATION

The product is not classified as dangerous according to EU Directives and national regulations. Contains harmful/irritant organic solvents.

3. COMPOSITION/INFORMATION ON INGREDIENTS

| Name | EC No. | CAS-No. | Content | Classification |
| --- | --- | --- | --- | --- |
| Alkanolamine |  |  | 0.1 – 2% | R36-48/22 Xn |
| Glycol solvent |  |  | 1 – 10% | R36 Xn |
| Glycol solvent |  |  | 1 – 10% | R22 Xn |
| Pyrrolidone solvent |  |  | 0 – 10% | R36/38 Xi |
| Dye component Chemical family | Magenta Azo | Yellow Azo | Cyan Copper phthalocyanine |  |

For full text see Section 16

EXPOSURE LIMITS (ACGIH 1993-1994)

American Conference of Governmental Industrial Hygienists

TLV: Threshold Limit Value TWA: Time Weighted Average STEL: Short Term Exposure Limit

CAS No.      Chemical Family		       TLV-TWA				 TLV-STEL

(ppm)         (mg/m3)		 (ppm) 	(mg/m3)

- -                 Alkanolamine                                                        5

- -                 Glycol solvent                                   c-39.4         c-100

c-indicates maximum exposure

4. FIRST-AID MEASURES

| # GENERAL INFORMATION | In all cases of doubt, or when symptoms persist, seek medical attention. Never give anything by mouth to an unconscious person |
| --- | --- |
| INHALATION | Remove to fresh air, keep the patient warm and at rest. If breathing is irregular or stopped, administer artificial respiration. Give nothing by mouth. If unconscious, place in the recovery position and seek medical advice. |
| INGESTION | If accidentally swallowed obtain immediate medical attention. Keep at rest. Do not induce vomiting. |
| SKIN CONTACT | Remove dirty clothing. Wash skin thoroughly with soap and water or use a proprietary skin cleaner. Do not use solvents or thinners. |
| EYE CONTACT | Contact lenses should be removed. Irrigate copiously with clean, fresh water for at least 10 minutes, holding the eyelids apart, and seek medical advice. |

5. FIRE-FIGHTING MEASURES

| # EXTINGUISHING MEDIA | Recommended: foam, CO2, powder, water, spray/mist. Not to be used: water jet. |
| --- | --- |
| SPECIAL FIRE FIGHTING PROCEDURES | Fire will produce dense black smoke containing hazardous products of combustion (see section 10). Decomposition products may be a hazard to health. Appropriate self-contained breathing apparatus may be required. Cool closed containers exposed to fire with water spray. Do not allow run-off from fire fighting to enter drains or watercourses. |
| UNUSUAL FIRE & EXPLOSION HAZARDS |  |
| SPECIFIC HAZARDS |  |

6. ACCIDENTAL RELEASE MEASURES

| # PERSONAL PRECAUTIONS | Exclude sources of ignition and ventilate the area. Exclude non-essential personnel. Refer to protective measures listed in Sections 7 and 8. |
| --- | --- |
| SPILL CLEAN UP METHODS | Contain and collect spillages with noncombustible absorbent materials, e.g. sand, earth, vermiculite, diatomaceous earth and place in a suitable container for disposal in accordance with the waste regulations (see Section 13). Do not allow to enter drains or watercourses. Clean preferably with a detergent; avoid the use of solvents. |

7. HANDLING AND STORAGE

| # USAGE PRECAUTIONS | Avoid vapor concentrations higher than the occupational exposure limits. Keep the container tightly closed. Exclude sources of heat and open flame. Avoid skin and eye contact. Smoking, eating and drinking should be prohibited in areas of storage and use. For personal protection, see Section 8. To avoid contamination always keep in containers made of the same material as the supply container. |
| --- | --- |
| STORAGE PRECAUTIONS | Store between 5 and 35 ºC in a dry, well ventilated place away from sources of heat, ignition and direct sunlight. No smoking. Prevent unauthorized access. Containers which are opened should be properly resealed and kept upright to prevent leakage. |

8. EXPOSURE CONTROLS/PERSONAL PROTECTION

| # STD | Users should take care about personal protection which is common using chemicals or chemical preparations (gloves, glasses). |
| --- | --- |
| ENGINEERING MEASURES | Provide adequate ventilation. |
| HAND PROTECTION | Rubber or PVC gloves |
| EYE PROTECTION | Eye protection designed to protect against liquid splashes should be worn. |

9. PHYSICAL AND CHEMICAL PROPERTIES

| APPEARANCE | Viscous liquid |  |  |
| --- | --- | --- | --- |
| COLOUR |  |  |  |
| ODOUR |  |  |  |
| SOLUBILITY | Completely miscible |  |  |
| MELTING POINT (°C) |  | BULK DENSITY |  |
| VAPOUR PRESSURE | Heavier than air | pH-VALUE, DILUTED SOLUTION |  |
| AUTO IGNITION TEMPERATURE (°C) |  |  |  |

10. STABILITY AND REACTIVITY

| # STABILITY | Stable under the recommended storage and handling conditions. (See section 7). In a fire, hazardous decomposition products such as smoke, carbon monoxide, carbon dioxide, sulfur dioxide and oxides of nitrogen may be produced. |
| --- | --- |
| HAZARDOUS DECOMPOSITION PRODUCTS | Keep away from oxidizing agents and strongly alkaline and strongly acidic materials to prevent the possibility of exothermic reaction. |

11. TOXICOLOGICAL INFORMATION

There is no data available on the product itself.

- -        Glycol solvent           LD50 (oral/rat)       >5000 mg / kg

- -        Alkanolamine            LD50 (oral/rat)       > 2000 mg / kg

- -        Glycol solvent            LD50 (oral/rat)       5840 mg / kg

- -        Pyrrolidone solvent   LD50 (oral/rat)       6500 mg / kg

| # GENERAL INFORMATION | There is no data available on the product itself |
| --- | --- |
| INHALATION |  |
| INGESTION |  |
| SKIN CONTACT |  |
| EYE CONTACT |  |

12. ECOLOGICAL INFORMATION

There is no data available on the product itself. The product should not be allowed to enter drains or watercourses or be deposited where it can affect ground or surface waters.

- -        Glycol solvent      Toxicity to fishes (Leucescus idus)             LC50      2200 – 4600 mg / l / 96h

- -        Glycol solvent      Toxicity to fishes (Pimephales promelas)    LC50       53000 mg / l / 96h

| # ECOTOXICITY | There is no data available on the product itself. |
| --- | --- |
| WATER HAZARD CLASSIFICATION | The product should not be allowed to enter drains or water courses or be deposited where it can affect ground or surface waters. |

13. DISPOSAL CONSIDERATIONS

| # GENERAL INFORMATION | The product (ink) should not be allowed to enter drains or watercourses or disposed of where ground or surface waters may be affected. Wastes, including emptied cartridges, are controlled wastes and should be disposed of n accordance with local regulations. |
| --- | --- |
| DISPOSAL METHODS |  |
| WASTE CLASS |  |

14. TRANSPORT INFORMATION

| # GENERAL | The product is not classified as dangerous for transport purposes. |
| --- | --- |

15. REGULATORY INFORMATION

This product is classified and labeled for supply in accordance with the regulations as follows:

The product is determined as not being dangerous.

The information contained in this safety data sheet does not constitute the user's own assessment of workplace risks as required by other health and safety legislation. No mark after decree of danger matter

| RISK PHRASES (EU) |  |  |
| --- | --- | --- |
| SAFETY PHRASES (EU) |  |  |
| REGULATORY REFERENCES (UK) |  |  |
| ENVIRONMENTAL LISTING (UK) |  |  |
| STATUTORY INSTRUMENTS (UK) |  |  |
| NATIONAL REGULATIONS (UK) |  |  |
| APPROVED CODE OF PRACTICE (UK) |  |  |
| GUIDANCE NOTES (UK) |  |  |
| DIRECTIVES (EU) |  |  |
| NATIONAL REGULATIONS (Other) |  |  |

16. OTHER INFORMATION

| # GENERAL INFORMATION | Text of R-phrases listed in Section 2: The product as contained within an inkjet cartridge and used according to normal usage, doesn’t present environmental or health risks. The cartridge should not be treated in such a way as no ink spell into the environment without first referring to the supplier and obtaining written handling instructions. As the specific conditions of use of this product are outside the supplier’s control, the user is responsible for ensuring that the requirements of relevant legislation are complied with. The information contained in this safety data sheet is based on the present state of knowledge and current national legislation. It provides guidance on health, safety and environmental aspects of the product and should not be construed as any guarantee of technical performance or suitability for particular applications. |  |
| --- | --- | --- |
| REVISION COMMENTS |  |  |
| REVISION / DATE |  |  |
| SDS NO. |  |  |
| RISK PHRASES IN FULL | R-phrases: R22. - Harmful if swallowed. R36. - Irritating to eyes. R36/38. - Irritating to eyes and skin. R48/22. - Harmful: danger of serious damage to health by prolonged exposure if swallowed. | Symbol letter: (Xn) HARMFUL (Xi) IRRITANT |

DISCLAIMER

This information relates only to the specific material designated and may not be valid for such material used in combination with any other materials or in any process. Such information is, to the best of the company's knowledge and belief, accurate and reliable as of the date indicated. However, no warranty guarantee or representation is made to its accuracy, reliability or completeness. It is the user's responsibility to satisfy himself as to the suitability of such information for his own particular use.