REVISION DATE:

SAFETY DATA SHEET

SPECIMEN SDS: Toner / Ink  for  BX - 3………………………………………………………………………………

1. IDENTIFICATION OF THE SUBSTANCE/PREPARATION AND OF THE COMPANY/UNDERTAKING

| # PRODUCT NAME | SPECIMEN SDS: Toner / Ink for |
| --- | --- |

| SUPPLIER | Environmental Business Products LTD | # MANUFACTURER |  |
| --- | --- | --- | --- |
| Street | 22 Abbey Rd, Park Royal, London | Street |  |
| Postal Code / City | NW10 7RE | Postal Code / City |  |
| Country | England | Country |  |
| Phone | 44 (0) 20 8838 6888 | Phone |  |
| ## SDS EMAIL CONTACT |  |  |  |
| EMERGENCY TELEPHONE |  |  |  |

# 2. HAZARDS IDENTIFICATION

This product is not classified as dangerous according to the regulations. It contains harmful/irritant organic solvents.

3. COMPOSITION/INFORMATION ON INGREDIENTS

| Name | EC No. | CAS-No. | Content | Classification |
| --- | --- | --- | --- | --- |
| Glycol Solvent |  |  | 0.5 – 10% | R22 Xn |
| Isopropanol |  | 67-63-0 | 0.5 – 10% | R11-36-67 Xi, F |
| Dye component Black |  |  |  |  |
| Chemical family Azo |  |  |  |  |

EXPOSURE LIMITS (ACGIH 1993-1994)

American Conference of Governmental Industrial Hygienists

TLV: Threshold Limit Value TWA: Time Weighted Average STEL: Short Term Exposure Limit

CAS No.                    Chemical Family		      TLV-TWA			      TLV-STEL

(ppm)	(mg/m3)		 (ppm) 	(mg/m3)

67-63-0                        Isopropanol 	     	         400	                  999	                   500	  1250

- -                              Glycol Solvent          	         C39.4           C100

C – Indicates Maximum Exposure

The Full Text for all R-Phrases are displayed in Section 16

4. FIRST-AID MEASURES

| # GENERAL INFORMATION | In all cases of doubt, or when symptoms persist, seek medical attention. Never give anything by mouth to an unconscious person. |
| --- | --- |
| INHALATION | Remove to fresh air, keep the patient warm and rest, if breathing is irregular or stopped, administer artificial respiration. Give nothing by mouth. If unconscious, place in the recovery position and seek medical advice. |
| INGESTION | If accidentally swallowed obtain immediate medical attention. Keep at rest. Do not induce vomiting. |
| SKIN CONTACT | Remove contaminated clothing. Wash skin thoroughly with soap and water or use a proprietary skin cleaner. Do not use solvents or thinners |
| EYE CONTACT | Contact lenses should be removed. Irrigate copiously with clean, fresh water for at least 10 minutes, holding the eyelids apart, and seek medical advice. |

5. FIRE-FIGHTING MEASURES

| # EXTINGUISHING MEDIA | Recommended : alcohol resistant foam, CO2, powder, water spray/mist |
| --- | --- |
| SPECIAL FIRE FIGHTING PROCEDURES | Not to be used: water jet |
| UNUSUAL FIRE & EXPLOSION HAZARDS | Fire will produce dense black smoke containing hazardous products of combustion (see section 10). Decomposition products may be a hazard to health. Appropriate self-contained breathing apparatus may be required. Cool closed containers exposed to fire with water spray. Do not allow run off from fire fighting to enter drains or watercourses |
| SPECIFIC HAZARDS |  |

6. ACCIDENTAL RELEASE MEASURES

| # PERSONAL PRECAUTIONS | Exclude sources of ignition and ventilate the area. Exclude non-essential personnel. Avoid breathing Vapours. Refer to protective measures listed in sections 7 and 8. |
| --- | --- |
| SPILL CLEAN UP METHODS | Contain and collect spillages with non-combustible materials, e.g. sand, earth, vermiculite, diatomaceous earth and place in a suitable container for disposal in accordance with the waste regulations (see section 13). Do not allow to enter drains or watercourses. Clean preferably with a detergent; avoid the use of solvents. |

7. HANDLING AND STORAGE

| # USAGE PRECAUTIONS | Avoid vapor concentrations higher than the occupational exposure limits. Keep the container tightly closed. Exclude sources of heat and open flame. Avoid skin and eye contact. Smoking, eating and drinking should be prohibited in areas of storage and use. For personal protection, see Section 8. To avoid contamination always keep in containers made of the same material as the supply container. |
| --- | --- |
| STORAGE PRECAUTIONS | Store between 5 and 35 ºC in a dry, well ventilated place away from sources of heat, ignition and direct sunlight. No smoking. Prevent unauthorized access. Containers which are opened should be properly resealed and kept upright to prevent leakage. Store separately from oxidizing agents and strongly alkaline and strongly acidic materials. |

8. EXPOSURE CONTROLS/PERSONAL PROTECTION

| # STD |  |
| --- | --- |
| ENGINEERING MEASURES | Provide adequate ventilation. Where reasonably practicable this should be achieved by the use local exhaust ventilation and good general extraction. If these are not sufficient to maintain concentrations of particles and/or solvent vapors below the relevant occupational exposure. |
| HAND PROTECTION | Rubber or PVC gloves. Barrier creams may help to protect exposed areas of the skin but are not substitutes for full physical protection. They should not be applied once exposure has occurred. |
| EYE PROTECTION | Eye protection designed to protect against liquid splashes should be worn. |

9. PHYSICAL AND CHEMICAL PROPERTIES

| APPEARANCE | Viscous liquid |  |  |
| --- | --- | --- | --- |
| COLOUR |  |  |  |
| ODOUR |  |  |  |
| SOLUBILITY | Complete |  |  |
| MELTING POINT (°C) |  | BULK DENSITY |  |
| VAPOUR PRESSURE |  | pH-VALUE, DILUTED SOLUTION |  |
| AUTO IGNITION TEMPERATURE (°C) |  |  |  |

10. STABILITY AND REACTIVITY

| # STABILITY | Stable under the recommended storage and handling conditions. (See section 7). In a fire, hazardous decomposition products such as smoke, carbon monoxide, carbon dioxide and oxides of nitrogen may be produced. |
| --- | --- |
| HAZARDOUS DECOMPOSITION PRODUCTS | Keep away from oxidizing agents and strongly alkaline and strongly acidic materials to prevent the possibility of exothermic reaction. |

11. TOXICOLOGICAL INFORMATION

| # GENERAL INFORMATION | There is no data available on the product itself - - Glycol Solvent LD50 5840 mg / kg (oral/rat) 67-63-0 Isopropanol LD50 >2000 mg / kg (rat/oral) |
| --- | --- |
| INHALATION |  |
| INGESTION |  |
| SKIN CONTACT |  |
| EYE CONTACT |  |

12. ECOLOGICAL INFORMATION

| # ECOTOXICITY | There is no data available on the product itself. - - Glycol Solvent Toxicity in fishes (Pimephales promelas) LC50 53000 mg / l / 96h 67-63-0 Isopropanol Toxicity in fishes LC50 9600 mg / l / 96h |
| --- | --- |
| WATER HAZARD CLASSIFICATION | The product should not be allowed to enter drains or water courses or be deposited where it can affect ground or surface waters. |

13. DISPOSAL CONSIDERATIONS

| # GENERAL INFORMATION | The product (ink) should not be allowed to enter drains or water courses or disposed of where ground or surface waters may be affected. |
| --- | --- |
| DISPOSAL METHODS |  |
| WASTE CLASS | Wastes, including emptied cartridges, are controlled wastes and should be disposed of in accordance with regulations |

14. TRANSPORT INFORMATION

| # GENERAL | The product is not classified as dangerous for carriage. |
| --- | --- |

15. REGULATORY INFORMATION

This product is classified and labeled for supply in accordance with the regulations as follows:

The product is determined as not being dangerous.

The information contained in this safety data sheet does not constitute the user's own assessment of workplace risks as required by other health and safety legislation.

| RISK PHRASES (EU) |  |  |
| --- | --- | --- |
| SAFETY PHRASES (EU) |  |  |
| REGULATORY REFERENCES (UK) |  |  |
| ENVIRONMENTAL LISTING (UK) |  |  |
| STATUTORY INSTRUMENTS (UK) |  |  |
| NATIONAL REGULATIONS (UK) |  |  |
| APPROVED CODE OF PRACTICE (UK) |  |  |
| GUIDANCE NOTES (UK) |  |  |
| DIRECTIVES (EU) |  |  |
| NATIONAL REGULATIONS (Other) |  |  |

16. OTHER INFORMATION

| # GENERAL INFORMATION | The product as contained within an inkjet cartridge and used according to normal usage, doesn’t present environmental or health risks. The cartridge should not be treated in such a way as no ink spell into the environment without first referring to the supplier and obtaining written handling instructions. As the specific conditions of use of this product are outside the supplier’s control, the user is responsible for ensuring that requirements of relevant legislation are complied with. The information contained in this safety data sheet is based on the present state of knowledge and current national legislation. It provides guidance on health, safety and environmental aspects of the product and should not be construed as any guarantee of technical performance or suitability for particular applications. |  |
| --- | --- | --- |
| REVISION COMMENTS |  |  |
| REVISION / DATE |  |  |
| SDS NO. |  |  |
| RISK PHRASES IN FULL | Text of R-phrases listed in Section 2 Symbol letter: (Xn) HARMFUL (Xi) IRRITANT (F) FLAMMABLE R-phrases: R11. - Highly flammable. R22. - Harmful if swallowed. R36. - Irritating to eyes. R67. - Vapours may cause drowsiness and dizziness. |  |

DISCLAIMER

This information relates only to the specific material designated and may not be valid for such material used in combination with any other materials or in any process. Such information is, to the best of the company's knowledge and belief, accurate and reliable as of the date indicated. However, no warranty guarantee or representation is made to its accuracy, reliability or completeness. It is the user's responsibility to satisfy himself as to the suitability of such information for his own particular use.